Name: NIVESH KUMAR V

Assignment no: 1

- OSI Layer of Wi-Fi

Wi-Fi, defined under IEEE 802.11 standards, operates in the lower two layers of the OSI model: the Physical Layer (Layer 1) and the Data Link Layer (Layer 2). At the Physical Layer, Wi-Fi is responsible for transmitting raw bits over the wireless medium using radio frequency signals. It defines modulation techniques such as OFDM, frequency bands (2.4 GHz, 5 GHz), and transmission power. At the Data Link Layer, specifically the MAC (Medium Access Control) sublayer, Wi-Fi manages how devices share the wireless medium. It uses CSMA/CA (Carrier Sense Multiple Access with Collision Avoidance) to reduce collisions, performs frame formatting, addressing using MAC addresses, and error detection. Thus, Wi-Fi ensures both signal transmission (Layer 1) and reliable communication (Layer 2).

- WiFi Devices and its capabilities

| Device | Wi-Fi Standard | Frequency | Max Speed | Features |
| --- | --- | --- | --- | --- |
| Smartphone | 802.11ac (Wi-Fi 5) | 2.4 + 5 GHz | ~866 Mbps | MU-MIMO, Beamforming |
| Laptop | 802.11ax (Wi-Fi 6) | 2.4 + 5 GHz | >1 Gbps | OFDMA, Target Wake Time |
| Router | Dual-band AC/AX | 2.4 + 5 GHz | 1200–3000 Mbps | Dual band, QoS |
| Smart TV | 802.11n | 2.4 GHz | ~150 Mbps | Basic streaming |

- A Basic Service Set (BSS) is the fundamental building block of a Wi-Fi network. It consists of a single Access Point (AP) and all the wireless devices connected to it. Each BSS is uniquely identified by a BSSID, which is the MAC address of the AP.

An Extended Service Set (ESS) is formed when multiple BSSs are interconnected through a distribution system (usually a wired LAN). All APs share the same SSID, allowing users to move seamlessly between coverage areas without losing connection. This process is known as roaming.

- A Wi-Fi Access Point acts as a bridge between wireless clients and a wired network. Its key functionalities include:

  - Providing wireless connectivity to devices

  - Broadcasting SSID for network identification

  - Managing authentication and security (WPA2/WPA3)

  - Assigning IP addresses via DHCP

  - Forwarding data between LAN and wireless devices

  - Selecting optimal channels to reduce interference

  - Supporting Quality of Service (QoS)

  - Enabling multiple device connections simultaneously

- Difference between Bridge mode and repeater mode

| Feature | Bridge Mode | Repeater Mode |
| --- | --- | --- |
| Function | Connects two networks | Extends Wi-Fi coverage |
| Medium | Wired/Wireless | Wireless only |
| Speed | No major loss | Speed reduces (50% typically) |
| Use case | Network expansion | Signal extension |

- Difference between 802.11a and 802.11b

802.11a operates in the 5 GHz band and supports higher data rates (up to 54 Mbps) with less interference. However, it has a shorter range due to higher frequency.

802.11b operates in the 2.4 GHz band and supports lower data rates (up to 11 Mbps) but offers better coverage and wall penetration. However, it is more prone to interference due to congestion in the 2.4 GHz band.

- IEEE is a standards organization that develops protocols like IEEE 802.11 for Wi-Fi communication.

The Wi-Fi Alliance (WFA) is an industry body that certifies devices for interoperability and markets Wi-Fi technologies (e.g., Wi-Fi 6).

- WiFi Backhaul

Backhaul refers to the connection between a Wi-Fi router/access point and the internet.

Common types include:

- Fiber (FTTH) – high speed and low latency

- DSL – moderate speed

- Cable – shared bandwidth

- Wireless (4G/5G hotspot)

Example:
A home network typically uses fiber backhaul providing high-speed internet with low latency.

- WiFi Topology and its test cases

| Topology | Description | Key Features | Use Cases |
| --- | --- | --- | --- |
| Infrastructure Mode | Devices connect through a central Access Point (AP) which acts as a bridge between wireless and wired networks. | Centralized control, easy management, supports internet access | Home networks, offices, colleges |
| Ad-hoc Mode (Peer-to-Peer) | Devices connect directly to each other without any Access Point. | No infrastructure required, simple setup, limited range | File sharing, temporary connections |
| Mesh Topology | Multiple interconnected APs (nodes) form a network providing seamless coverage. | Self-healing, wide coverage, no single point of failure | Smart homes, campuses, large buildings |
| Point-to-Point (P2P) | Direct wireless connection between two fixed locations. | Long-distance communication, high speed, dedicated link | Building-to-building connection, CCTV links |
| Point-to-Multipoint (P2MP) | One central AP connects to multiple client devices simultaneously. | Efficient distribution, scalable, centralized control | ISP networks, rural internet distribution |

| Parameter | 2.4 GHz | 5 GHz |
| --- | --- | --- |
| Frequency Band | 2.4 GHz | 5 GHz |
| Signal Strength | Strong | Medium |
| Link Speed | Lower | Higher |
| Download Speed | Moderate | High |
| Upload Speed | Moderate | High |
| Ping | Higher | Lower |
| Range | Longer | Shorter |
| Interference | High | Low |