Name: NIVESH KUMAR V

Assignment no: 5

1. Key features of Wi-Fi 6, 6E, and 7 & difference from Wi-Fi 5

- Wi-Fi 5 (802.11ac):
Operates only in 5 GHz, uses OFDM and supports downlink MU-MIMO. Focus is mainly on increasing peak data rate.

- Wi-Fi 6 (802.11ax):
Operates in 2.4 & 5 GHz. Introduces OFDMA, uplink & downlink MU-MIMO, 1024-QAM, and Target Wake Time (TWT). Focus is on efficiency and handling dense networks.

- Wi-Fi 6E:
Extends Wi-Fi 6 into the 6 GHz band, providing wider channels, more spectrum, and less interference.

- Wi-Fi 7 (802.11be):
Introduces 320 MHz bandwidth, 4096-QAM, Multi-Link Operation (MLO), and enhanced scheduling. Focus is on ultra-high throughput and low latency applications.

2. Role of OFDMA in Wi-Fi 6

OFDMA divides a wide channel into smaller Resource Units (RUs), allowing multiple users to transmit simultaneously within the same channel.

- Reduces contention among users

- Improves spectral efficiency

- Lowers latency

- Supports high-density environments

Unlike OFDM (single user), OFDMA enables multi-user parallel transmission.

3. Benefits of Target Wake Time (TWT)

TWT allows devices to negotiate specific wake-up times with the access point.

- Reduces idle listening → saves battery

- Minimizes collisions by scheduling transmissions

- Improves network efficiency

- Particularly useful for IoT devices and low-power sensors

4. Significance of 6 GHz band (Wi-Fi 6E)

The 6 GHz band provides a new, clean spectrum for Wi-Fi.

- Large contiguous bandwidth

- More non-overlapping channels (20/40/80/160 MHz)

- Minimal interference from legacy devices

- Supports high throughput and low latency

Ideal for high-bandwidth applications like AR/VR and 4K/8K streaming.

5. Wi-Fi 6 vs Wi-Fi 6E

| Feature | Wi-Fi 6 | Wi-Fi 6E |
| --- | --- | --- |
| Frequency Bands | 2.4 & 5 GHz | 2.4, 5 & 6 GHz |
| Spectrum Availability | Limited | Expanded |
| Interference | Moderate | Very low |
| Channel Width | Up to 160 MHz | More clean 160 MHz channels |
| Performance | High | Higher due to less congestion |

6. Major innovations in Wi-Fi 7 (802.11be)

- 320 MHz channels → doubles bandwidth

- 4096-QAM → increases bits per symbol

- Multi-Link Operation (MLO) → simultaneous multi-band usage

- Enhanced MU-MIMO and OFDMA

- Lower latency and jitter

Designed for real-time applications like gaming, AR/VR, and industrial control.

7. Multi-Link Operation (MLO)

MLO allows a device to use multiple frequency links (2.4, 5, 6 GHz) simultaneously.

- Parallel data transmission

- Improved throughput

- Reduced latency

- Better reliability (failover capability)

Overcomes limitations of single-band communication.

8. Purpose of 802.11k and 802.11v

- 802.11k (Radio Resource Management):
Provides information about neighbouring APs (signal strength, load), helping clients make better roaming decisions.

- 802.11v (Network Assisted Roaming):
Allows APs to suggest better APs to clients for improved load balancing and performance.

9. Fast BSS Transition (802.11r)

802.11r reduces roaming delay by enabling fast re-authentication.

- Pre-establishes security keys

- Eliminates full authentication during handoff

- Reduces latency significantly

Critical for real-time applications (VoIP, video calls).

10. How 802.11k/v/r work together

These standards work together to provide efficient and seamless roaming:

- 802.11k: Supplies neighbour AP information

- 802.11v: Guides client to optimal AP

- 802.11r: Enables fast and secure transition

Combined effect:

- Faster roaming

- Reduced packet loss

- Improved user experience